Device: Jetson nano

Language: Python with threading

Requirements

When the device program starts, it reads settings from local Json file and updates the settings variables

The device will send continuously every 2 second to get commands.

The device can respond to 3 commands start, stop and update settings (confidence threshold).

The device will store settings in Json file.

Stop command

when device receives stop command, it stops the weed detection sequence abruptly.

Update setting command

When the device receives this command, the device saves the settings in Json file and update variables in program.

Start command

This command has parameter travel distance and will start weed detection sequence.

weed detection sequence

Step1: Take photo from USB camera and store the photo.

Step2: Run yolo.pt model to detect weed.

Step3: draw the bounding box for detected weed and save the image.

Step4: create a Json file with detected object coordinates, class label and confidence score.

Step5: Upload the raw image, image with bounding box and Json file to server and delete the local copy.

Step6: Send move forward command with distance(meters) via usb com port to nodeMCU.

Step7: after moving MCU send command back via usb com port that position is reached.

Step8: If total distance is covered send completed http request to web server, otherwise repeat step1 to step6.